| Assumptions and introductory remarks Transmodel uses the term ‘modes of operation’ and the following categorization: • Conventional modes (of operation): o Scheduled o Flexible • Alternative modes (of operation): o Vehicle pooling o Vehicle rental o Vehicle sharing o Taxi o Personal modes (of operation) Demand-responsive transport modes according to the terminology of DR (EU) 2017/1926 (or on-demand as appear on amendment DR (EU) 2024/490) are associated in this document with flexible and alternative modes (of operation) according to the terminology of Transmodel. Abbreviations Abbreviation Meaning OJP Open Journey Planner OSM OpenStreetMap OTP OpenTripPlanner POI Point of Interest TSC Transport Service Carrier TSP Transport Service Provider WGS84 World Geodetic System 1984 | N.v.t. |
| --- | --- |
| Types of Static, Historical, and Observed Travel and Traffic Data | N.v.t. |
| Level of Service 1 (data regarded as essential for the basic functioning of multimodal travel information services) | N.v.t. |
| Location search (origin/destination) – addresses (building number, street name, postcode): the minimum information required for matching specific locations (mainly properties) as trip origins and destinations based on their address. The following data needs to be provided for an address: • Identifier (must be globally unique) • Name • Coordinates (Centroid) • Topographic or address identifiers • Category of classification The following data is optional for an address: • External identifiers (e.g., for OpenStreetMap (KeyList) • Additional coordinates (access) • Area • Validity | Abstract objecttype Locatie met attributen identificatie, naam, geometrie (punt) en gegevensgroep tijdvakGeldigheid met attribuutsoorten begindatum en optioneel einddatum. Objecttype Adres is een specialisatie van Locatie. Topografische of adres identificaties en categorie volgen uit de specialisaties. |
| Correspondence with Transmodel/NeTEx Concepts ROAD ADDRESS :: Specialization of ADDRESS using conventional characteristics such as road number and name for identification of a location along a road. NeTEx POSTAL ADDRESS :: A specification of ADDRESS refining it by using the attributes used for conventional identification for mail. Comprises variously a building Identifier, Street name, Post code, and other descriptors. | Objecttype Wegadres als specialisatie van Adres met optionele attributen wegnaam, en wegnummer. Objecttype Postadres als specialisatie van Adres met verplichte attributen straatnaam, huisnummer, postcode, plaats, en land. Attribuut gebouwidentificatie volgt uit relatie met IMBAG:AdresseerObject. |
| BEST PRACTICES • Addresses that appear in this data type are mostly relevant to the postal address. • Examples of this are: building numbers, street numbers, postcodes linked with coordinates or other location references (coordinates will be supplied according to WGS84). • When talking of the origin and destination of a trip, it should be made clear if the reference for origin and destination is the physical location of the departure or arrival of the customer’s trip (e.g. from home and to a shopping mall), or the public transport point of origin and destination. In the latter case you need to add information about an access and egress trip leg which is most often not done by public transport (except if you order a taxi or an Uber-like service or some on-demand shuttles). • Based on INSPIRE data specification, the postal address constitutes a hierarchy consisting of components with an increasing level of detail, e.g., town, street name, and house number or house name. It may also include a post code or other postal descriptors. Both types of addresses may be used: o The reference for postal address format is INSPIRE. o The reference for road address is NeTEx. References/additional info: EN12896-1 (see https://www.transmodel-cen.eu); CEN/TS 16614-1 (see https://netex-cen.eu); https://publications.jrc.ec.europa.eu/repository/handle/JRC118744 | N.v.t. |
| Location search (origin/destination) – topographic places (city, town, village, suburb, administrative unit): the minimum information required for matching wider locations (e.g., areas, regions, localities, cities, suburbs, towns, administrative units, or settlements) as trip origins and destinations based on their name. The names of settlements, counties, etc., are of importance for locating trip endpoints, namely for the distinction of place names that are not globally unique. The following data needs to be provided for topographic places: • Identifier (must be globally unique) • Name • Coordinates (Centroid) • Topographic or address identifiers • Category or classification The following data is optional for an address: • External identifiers (e.g., for OSM) (KeyList) • Area • Validity | Objecttype Plaats als specialisatie van Locatie. |
| Correspondence with Transmodel/NeTEx Concepts TOPOGRAPHIC PLACE A type of PLACE providing the topographical context when searching for or presenting travel information, for example as the origin or destination of a trip. It may be of any size (e.g., County, City, Town, Village) and of different specificity (e.g., Greater London, London, West End, Westminster, St James’s). Based on the Transmodel/NeTEx data specification, TOPOGRAPHIC PLACEs may be located within one single country, may intersect one or more countries, and may be contained inside other topographic places. TOPOGRAPHIC PLACEs can include disambiguation information to ensure that different places with the same name can be distinguished when searching for place names. References/additional info: EN12896-1 (see https://www.transmodel-cen.eu); CEN/TS 16614-1 (see https://netex-cen.eu); https://publications.jrc.ec.europa.eu/repository/handle/JRC118744 | Definitie en toelichting van objecttype Plaats is gebaseerd op deze nadere duiding. |
| Location search (origin/destination) – points of interest (related to transport information) to which people may wish to travel: the minimum information required for matching specific locations of interest (e.g., amenities, landmarks, tourist attractions) as trip origins and destinations (covers DR(EU)1926/2017 1.b.c). The coordinates and/or address identifiers of any point of interest (POI) that are relevant to travellers. For example, a museum, a theatre, or a shopping mall. The following POI should be (at least) provided: • Administrative buildings • Airports • Amenities (such as shopping mall, restaurants) • Hospitals • Museums, tourist attractions • Pharmacies • Police stations • Ports • Protected sites • Rental stations • Universities, schools | Objecttype Locatie van belang als specialisatie van Locatie. Objecttype Locatie van belang heeft een attribuut type met enumeratie TypeLocatieVanBelang met waarden |
| BEST PRACTICES Be aware that there might exist temporary POI as well (e.g. a circus). Be aware that some locations are considered as input to trip planners but are not POI in the narrower sense. They are also not modelled as POI, but should still contain the relevant information in their respective data types (e.g. stations, charging stations, parking): • Parking (see section on parking) • Park & Ride (all vehicles), kiss & ride: covered under “location search – for transport on demand and personal transport: Park & Ride stops” • Heavy vehicle parking: covered under DR EU 885/2013 “provision of information services for safe and secure parking places for trucks and commercial vehicles” • Charging stations, gas stations (personal operated vehicles): covered under DR EU 2022/670 “provision of EU-wide real-time traffic information services” • Public transport stops (see section on stops) | Objecttype Parkeerplaats en Park & Ride als objecttype opgenomen in Location search. Nadere uitwerking volgt. |
| PROCEDURES • The data must follow NeTEx Part 4 (EN 12896-4, European Passenger Information Profile, EPIP) in the structure for the data export. • An export as Open Journey Planner (OJP) Location Information is also possible (TS17118, version 2). • The data must be actualized at least once a year. In the ideal case any update to a POI (content, existence, coordinates) should be available on the following day. • The POI are synchronized with the ones in readily available geobases (e.g. OpenStreetMap) and the national geobases. | TODO |
| DATA STRUCTURE The following data define a point of Interest: • Mandatory: o Identifier (must be globally unique): The identifier that is used afterwards in all other data. o Name o Coordinates (Centroids): A version in WGS84 is mandatory. A local projection can be added. o Category or classification: There can be multiple classification systems used. It is recommended that at least the OSM map feature classification1 is supported. • Recommended: o External references (e.g., for OSM or a national GeoBase): This helps in using the POI in the diverse world we have. o Source: A reference to the source of the data. o Topographic or address identifiers: If such id exist, they should be provided. o Access and egress points to the road network (doors and gates) – with coordinates: For bigger structures they are needed. • Optional: o Area: Some POIs are big and the area covered is of importance. o Validity: If a POI is valid only for a given period. This data can be provided. o Opening hours | TODO |
| IMPLEMENTING SERVICES • Journey Planners • Geocoders RELEVANT STANDARDS & LEGISLATION • EU/1926/2017 • EU/885/2013 • EU/40/2010 | TODO |
| Correspondence with Transmodel/NeTEx Concepts POINT OF INTEREST A type of PLACE to or through which passengers may wish to navigate as part of their journey and which is modelled in detail by journey planners. Transmodel/NeTEx provides a model for a generalized classification system that allows a point of interested to be assigned to one or more categories (POINT OF INTEREST CLASSIFICATIONs) and the categories themselves to be organized into hierarchies (POINT OF INTEREST CLASSIFICATION HIERARCHY). References/additional info: EN12896-2 (see see www.transmodel-cen.eu); CEN/TS 16614-1 (see https://netex-cen.eu); https://publications.jrc.ec.europa.eu/repository/handle/JRC1187443 [3] This report clarifies that INSPIRE can be considered as reference standard only for a limited set of POI categories (Utilities and Governmental Services (US), Buildings (BU), and Protected Sites (PS)), each of which has its own specific data model. ? |  |
| Trip plans – operational calendar, mapping day types to calendar dates: the minimum information required to indicate the days or periods when transportation services are operational. The operational calendar describes the specific days of operation for public transport services (days, period, day types). For example, it can describe that a specific line operates on all weekdays (Monday to Friday) except for specific public holidays (e.g., December 25). Correspondence with Transmodel/NeTEx Concepts SERVICE CALENDAR A collection of assignments of operational characteristics (expressed by DAY TYPEs characterised by different properties of days, for example weekday in school holidays) to particular OPERATING DAYs. NeTEx POSTAL ADDRESS A day of public transport operation of which the characteristics are defined within a specific SERVICE CALENDAR. An OPERATING DAY may last more than 24 hours. A DAY TYPE ASSIGNMENT relates an OPERATING DAY to a DAY TYPE. References/additional info: EN12896-1 (see https://www.transmodel-cen.eu); CEN/TS 16614-1 (see https://netex-cen.eu) | TODO |
| Location search (access nodes) – for scheduled transport and transport on demand where relevant - identified access nodes: the minimum information required to identify the location of stop or other specific places from which travellers can access scheduled or on-demand transport modes. Correspondence with Transmodel/NeTEx Concepts SCHEDULED STOP POINT A POINT where passengers can board or alight from vehicles. STOP PLACE A place comprising one or more locations where vehicles may stop and where passengers may board or leave vehicles (QUAYs) or prepare for their trip. The STOP PLACE is entered through designated ENTRANCEs and may also contain ACCESS SPACEs (locations passed through to reach the QUAYs). A STOP PLACE will usually have one or more well-known names. | Abstract objecttype Toegangspunt (alias Access Node) met relatie 'heeft' Locatie. Objecttype Gepland stoppunt (alias Scheduled Stop Point) als specialisatie van Toegangspunt. Objecttype Halteplaats als specialisatie van Toegangspunt met relatie 'bestaat uit 'één of meer Geplande stoppunten, een relatie 'is toegankelijk via' nul of meer Ingangen, en een relatie 'biedt verplaatsingsruimte via' nul of meer Doorgangen. Objecttype Ingang met attributen n.t.b.; objecttype Doorgang (alias Access Space) met relatie 'biedt toegang tot' nul of meer Perrons. Objecttype Perron (alias Quay) met relatie 'bedient' één of meer Geplande stoppunten. Objecttype Ingang, Doorgang en Perron hebben een relatie 'heeft als locatie' één of meer Locaties. |
| BEST PRACTICES • From a topological point of view, access nodes for public transport are zero-dimensional nodes of the transport networks (road, rail, etc.), that may be located by coordinates in a particular Coordinate Reference System, used for the spatial description of the network, where passengers may board or alight from vehicles. • From a geographical point of view, access nodes are places that appear in the timetable as locations where vehicles may stop, and passengers may board or alight from vehicles. References/additional info: EN12896-2 (see https://www.transmodel-cen.eu); CEN/TS 16614-1 (see https://netex-cen.eu); https://publications.jrc.ec.europa.eu/repository/handle/JRC118744 | TODO |
| "Location search (access nodes) – for scheduled transport and transport on demand where relevant - geometry/map layout structure of access nodes: the minimum information required for identifying the location of specific facilities within access nodes considering the topographical structure of access nodes. Correspondence with Transmodel/NeTEx Concepts EQUIPMENT PLACE :: A SITE (e.g., a STOP PLACE) COMPONENT containing EQUIPMENT. EQUIPMENT POSITION :: The precise position within an EQUIPMENT PLACE where particular equipment is placed." It is also possible to relate equipment to PATH LINKs; PATH LINKs within a STOP PLACE are connecting between components. (e.g., a STOP PLACE ENTRANCE, ACCESS SPACE, QUAY, BOARDING POSITIONs, etc.), or PATH JUNCTIONs (i.e. points at which two or more PATH LINKs may connect or branch). Equipment may be represented on a SCHEMATIC MAP: A map representing schematically the layout of the topographic structure of PLACEs (e.g., a set of SITEs) or the public transport network (a set of LINEs), or other entity with a geometric projection (e.g., DECK PLAN for a train, vessel or aircraft). It can include a pixel projection of a set of ENTITies onto a bitmap image so as to support hyperlinked interactions. The nature of amenities available at an access node can be specified as FACILITies and LOCAL SERVICEs. Further detailed information can be provided using EQUIPMENT of different types (e.g., LIFT EQUIPMENT, TICKET EQUIPMENT, ENTRANCE EQUIPMENT, etc.). Within an access node EQUIPMENT can be exactly located using an EQUIPMENT POSITION. The topographical structure of an access node can be described through the spatial layout of its subcomponents and by explicit links between them, over which a trip planner may compute a route through a site. The plan of a STOP PLACE and the paths through it can be visualized on a digital map. References/additional info: EN12896-2 (see https://www.transmodel-cen.eu); CEN/TS 16614-1 (see https://netex-cen.eu); https://publications.jrc.ec.europa.eu/repository/handle/JRC118744 | Objecttype Apparatuurplaats (alias Equipment Place) met relatie 'bevindt zich' in één Halteplaats. Objecttype Apparaat (alias Equipment) met attribuut type met classificaties informatiescherm, lift, roltrap, ticketapparaat, toegangspoort, veiligheidsvoorziening, verlichting, en de relatie 'bevindt zich' in één Apparaatuurplaats en relatie 'is gepositioneerd in' naar gegevensgroeptype Apparaatpositie (alias Equipment Position). Objecttype Routeverbinding (alias Path Link) met als relatie 'ligt in' één Doorgang en relaties 'heeft als beginpunt' en 'heeft als eindpunt' naar objecttype Routeknooppunt (alias Path Junction) met als rol respectievelijke beginknoop en eindknoop. Objecttype Instappositie (alias Boarding Position) met relatie 'realiseert' één of meer Geplande stoppunten en relatie 'ligt aan' Routeverbinding. Objecttype Voorziening (alias Facility) en Lokale dienst (alias Local service) met relaties van Toegangspunt naar 'voorziet in' Voorziening en biedt aan' Lokale dienst. |